entity switch is

port (data_in : in bit_vector (0 to 3);

data_out : out bit_vector (0 to 3);

data_inavail : in bit_vector (0 to 3);

data_outavail : in bit_vector (0 to 3);

CLK : in bit);

end switch;

entity inport is

port (next_data : in bit;

address0 : out bit;

address1 : out bit;

packet_ready : out bit;

current_data_bit : out bit;

data_in : in bit;

data_avail : in bit;

CLK : in bit);

end inport;

architecture behavior of inport is

begin

process (CLK)

variable count : integer :=0;

variable bits_remaining : integer;

variable shift_register : bit_vector (0 to 5);

begin

if CLK = '1' then

if next_data = '1' then

shift_register (3) := shift_register (2);

shift_register (2) := shift_register (1);

shift_register (1) := shift_register (0);

bits_remaining := bits_remaining - 1;

if bits_remaining = 0 then

count := 0;

packet_ready <= '0';

end if;

end if;

if data_avail = '1' and count < 6 then

shift_register (5) := shift_register (4);

shift_register (4) := shift_register (3);

shift_register (3) := shift_register (2);

shift_register (2) := shift_register (1);

shift_register (1) := shift_register (0);

shift_register (0) := data_in;

count := count + 1;

if count = 6 then

bits_remaining := 4;

packet_ready <= '1';

end if;

end if;

address0 <= shift_register (4);

address1 <= shift_register (5);

current_data_bit <= shift_register (3);

end if;

end process;

end behavior;

architecture behavioral of switch is

component inport

port (next_data : in bit;

address0 : out bit;

address1 : out bit;

packet_ready : out bit;

current_data_bit : out bit;

data_in : in bit;

data_avail : in bit;

CLK : in bit);

end component;

signal next_data : bit_vector (0 to 3);

signal data_bit : bit_vector (0 to 3);

signal address0 : bit_vector (0 to 3);

signal address1 : bit_vector (0 to 3);

signal p_ready : bit_vector (0 to 3);

begin

b0 : inport port map (next_data (0), address0 (0), address1 (0), p_ready(0),

data_bit (0), data_in (0), data_inavail (0), CLK);

b1 : inport port map (next_data (1), address0 (1), address1 (1), p_ready(1),

data_bit (1), data_in (1), data_inavail (1), CLK);

b2 : inport port map (next_data (2), address0 (2), address1 (2), p_ready(2),

data_bit (2), data_in (2), data_inavail (2), CLK);

b3 : inport port map (next_data (3), address0 (3), address1 (3), p_ready(3),

data_bit (3), data_in (3), data_inavail (3), CLK);

process

variable add :integer;

variable i :integer := 0;

begin

wait until CLK'event and CLK = '1';

add:=0;

if p_ready(i) = '1' then

if address0(i) = '1' then add:=add+1; end if;

if address1(i) = '1' then add:=add+2; end if;

data_out(add) <= data_bit(i);

next_data (i) <= '1';

wait until CLK'event and CLK = '1';

wait for 0ns;

data_out(add) <= data_bit(i);

wait until CLK'event and CLK = '1';

data_out(add) <= data_bit(i);

wait until CLK'event and CLK = '1';

data_out(add) <= data_bit(i);

end if;

i := i+1;

if i = 4 then i = 0; end if;

end process;

end behavioral;